ECE 4380 – 001

Embedded System’s Capstone

Brance Boger

Keven Cruz

Robert Inyang

Texas Tech University

Electrical and Computer Engineering Department

December 9, 2025

Abstract

The aim of this project is to design and implement an RTOS system using an ARM Cortex-M microcontroller. This project demonstrates the design and development of a Smart Apartment Energy Monitoring and Control System based on an STM32 Nucleo-F411RE board, where a student-developed full real-time operating system (RTOS) runs along with the following components: two INA219 current/voltage sensors to measure the energy consumed by any USB-connected devices, an ESP-01S Wi-Fi module acting as a standalone access point, and a 0.96-inch OLED display to show local readings. Differently from the classical domestic energy monitoring solutions where the processing and data management tasks are performed in the cloud and managed by proprietary software, in our project the focus is on transparency and predictability thanks to the implementation of a student-developed scheduling kernel able to handle three real-time tasks: energy sampling, data update to the display, and system heartbeat.

Acknowledgements

Special thanks to my partners for helping me on this project, I wouldn’t have been able to get any of this done without them.

Table of Contents

Abstract	ii

Acknowledgements	iii

Table of References	iv

Introduction	1

STM32-Based System Core and Custom RTOS	1

INA219 Current and Voltage Monitoring	2

OLED Display Module	2

ESP-01S Wi-Fi Access Point Module	3

Conclusions	3

Introduction

The purpose of this project is to design and implement a compact, economical, and flexible smart apartment energy monitoring system that allows users to analyze the energy utilization of normal USB-operated devices. The design focuses on the STM32 Nucleo-F411RE board because of its performance and suitability for normal low-level embedded programming and development. This project will serve to highlight how a normal commercial smart socket and cloud-based energy solution compares to a student-developed and open-source based energy solution that directly exposes the users to the energy measurements and aspects involved. The design encompasses real-time current measurement, LCD messaging, wireless communication functionality, and an RTOS for handling multiple tasks.

STM32-Based System Core and Custom RTOS

The STM32 Nucleo-F411RE functions as the processing center where all the time-critical operations are performed through a complete student-developed RTOS. The RTOS has a deterministic scheduling algorithm implemented through a millisecond resolution SysTick interrupt and a cooperative process execution pattern based on a lightweight embedded RTOS design. The RTOS allows for the implementation of multiple periodic tasks, inter-process communications through a software-based message queue, and context-dependent updates of peripherals such as I²C sensors and the OLED display. Through the absence of any RTOS libraries, the firmware allows complete insight into process and interrupt handling, and execution deadlines, a critical requirement for real-time measurements of energy.

INA219 Current and Voltage Monitoring

There are two INA219 current sensors that serve as the data-acquisition components in the system, allowing real-time current and voltage monitoring of the two USB-connected devices. Each of the two INA219s has an I²C interface and measures the voltage across a precise resistor in the circuit. The system initializes and calibrates the two INA219s and samples the data at an interval of every 100 ms to calculate the real-time watts and watt-hours. The system shows an accuracy of ±5% in its measurements, meeting the project requirement for resolution in the measurements made by the meter.

OLED Display Module

For local real-time monitoring of the system’s power information, an SSD1306-based OLED module with a resolution of 0.96 inches is used. The OLED module displays the value of the channel powers, voltage, current, and total energy in terms of watt hours. The rendering engine utilizes an optimized algorithm for writing texts, which is directly programmed in the C language. The RTOS has a specific task named “Display update task” to update the display at a rate of every 200 ms to avoid any interference with the critical process of INA219 sampling.\

ESP-01S Wi-Fi Access Point Module

The ESP-01S module is designed to be a standalone access point, and it establishes its own Wi-Fi network to which the smartphone will be connected for possible future expansion to web-based dashboards. The STM32 module interacts with the ESP-01S module through the UART interface, and communications occur through the AT command protocol and setting the SSID, password, and channel number.  This device allows the user to be able to access the smart energy system anywhere within the Wi-Fi range of the ESP-01S.

Conclusions

The project shows how a smart-home energy monitoring system, utilizing an STM32 microcontroller, an RTOS designed and developed in the project, and low-cost sensors, can be implemented. The system has the basic elements of live readings, real-time monitoring, and the provision to access the readings through a wireless connection and has the scope to be expanded into a more complex system, such as automated load shedding and cloud-based monitoring systems based upon machine learning algorithms.